# 1 COMPANY DESCRIPTION

## 1.1 Name of the Institute

### TUBITAK – BILTEN

## 1.2 Location of the Institute

### METU – ANKARA

## 1.3 Brief History of the Institute

Middle East Technical University (METU) and Turkish Scientific and Technical Research Council (TÜBİTAK) have signed a protocol on 30.10.1984 to establish an institute on electronics in the campus of METU. The related regulation was published in the State Official Gazette on 15.02.1985 and Ankara Electronics Research and Development Institute was founded and located at the Department of Electrical and Electronics Engineering. The organizational structure of the Institution has been dramatically changed since 14.6.1991 and the project groups have become the main structural part of the organization. On 26.5.1995, the Institute’s working field was extended by a change in its Regulation and Institute’s name was changed as Information Technologies and Electronics Research Institute (BİLTEN).

## 1.4 Missions of the Institute

Information technologies include data gathering, data processing, data organizing, data storage and access, and all other related technologies. Therefore, the major interests of the Institute are in the fields of electrical and electronics engineering, computer science and engineering and, industrial engineering. The Institute carries out research and studies mainly on computer hardware and software, electronics and communications, image processing and coding, the power and control systems related to the information technology. There are 16 project groups working on these topics in the Institute, currently.

The Institute carries out its activities according to the principles stated as follows:

- To define short and long term research targets and problems related to the Institute’s responsibilities and to solve or assist in solving those problems by taking into account the national development objectives and, developments taking place in the world.

- To carry out all research, development and consultant services in the areas covered by its objectives and required by public or private sector under a contract; to contribute for an improvement of research capabilities in Turkey; to cooperate with universities and other research institutes in development and exchange of projects in order to achieve the foregoing objectives; to take part in international projects.

- To carry out all research and development studies from prototype production to initial production stage of component, devices, software and systems in cooperation with other institutions when required and to prepare implementation plans, technical data and documents required for mass production,

- To provide equipment and materials which are difficult to install and operate where required and, to help in installation and operation of them at research and training institutes providing an access to local researchers, to develop materials and equipment required for researches within the Institute, to set up pilot systems,

- To prepare proposals to guide activities being carried out in respective fields in Turkey and to ensure coordination between such activities, to follow technological developments abroad and to investigate possibility of application of such technologies and function as a reference center in the country, to carry out necessary studies in the modification of imported technologies in order to adapt them to local conditions and to carry out and/or have such activities carried out under contracts,

- To assist public and private organizations in solving technical problems relating to design, selection, application of systems and product development and production,

- To organize seminars, symposiums, congress, lectures etc. on the areas of interest.

## 1.5 Brief Information about the Satellite Technologies Group

Main aim is to initiate a systematic production of Low-Earth-Orbit (LEO) Micro-and Mini -Satellites in Turkey. By this endeavor we hope to enable Turkey to join the international market for remote-sensing imaging as a supplier notion. Group predominantly focus on the "TÜBİTAK-BİLTEN Research Satellite Development Project" designed to produce a micro- or mini-satellite system. The primary purpose of it is to perform remote-sensing operations over Turkey and the Central Asian Republics and to control the satellite system via a number of low-cost ground stations located in the region of interest.  It is planning to get involved in joint research projects in this area with national and international partners.

# 2 INTRODUCTION

The main aim of the summer practice was to gain experience as an aeronautical engineer in the industry, and getting familiar with the environment of business life and employee relationship.

The scope of the summer practice was on understanding of satellite dynamics and control systems of it. In order to getting familiar with satellites brief information about orbital mechanic such as orbit types, maneuvers ...etc and attitude control devices such as thrusters, reaction wheels ...etc were obtained.

After studying on the dynamic equations of single and three reaction wheeled conceptual satellites and the behaviors of them with a given disturbances were simulated and the appreciate control system was adapted to the system.

In the summer practice the software packages mostly used were MATLAB and MAPLE scientific computing tools, Satellite Tool Kit and SOAP orbital simulation tools.

This report includes brief information about the institute and the out come of the six weeks summer practice.

# 3 DYNAMIC EQUATIOS OF THE SINGLE WHEELED CONCEPTUAL SATELLITE

## 3.1 Reference Frames

In order to set up the systems dynamic equations first the proper coordinate frames must be chosen. In this study three coordinate systems were chosen.

Z2

Y2

X2

Z1

X0

Y1

Z0

X1

Y0

Earth Center

Satellite

Orbit

Fig2 1 Reference Frames

The first coordinate system is the absolute (inertial) frame, which is indicated by “X0 Y0 Z0” (It will be referred as frame [0] in rest of the text), in Fig 2-1. The second coordinate system is the indicated by “X1 Y1 Z1” (It will be referred as frame [1] further in the text) in Fig 2-1. Both of the coordinate systems have an origin at the center of the earth, but frame [0] is fixed in space whereas frame [1] is rotating around Y1 with fixed orbital rate. The third coordinate system is denoted by “X2 Y2 Z2” (It will be referred as frame [2] further in the text) in Fig 2-1. The origin of frame [2] is at the mass center of the satellite and fixed according to the satellite.

## 3.2 Satellite Model

The satellite is a cube and homogeneous mass distribution shown in Fig 2-2. Roll, pitch, yaw axes are denoted by X2 Y2 Z2, respectively and rotations about them referred by respectively. The momentum wheel is located at the mass center of the wheel and its axis of rotation is aligned with Y2.Two couples of thrusters are mounted on the sides of the satellite in order to produce torque about roll and yaw axes.

Y2 (pitch)

Z2 (yaw)

X2 (roll)

Momentum Wheel

Thrusters

Fig 2-2 Satellite Model

## 3.4 Equations of Motion

From the relation between angular momentum and external torque it can be written:

M2=ddth2

Where

M2 sum of all torques acting about origin of frame[2]h2 sum of all angular momentums acting about origin of frame[2]

Since there is both satellite and the reaction wheel exists angular momentum composed of summation of two.

h2=hsat+hwheel

For Satellite only;

hsat=Isatωsat

In body frame

Ix 0  00  Ix 0

D0hsat=D2hsat+ω×hsat

D0 is the derivation with respect to time in frame [0].

D2 is the derivative with respect to time in frame [2].

hsat=hsatbody+ω⋅hsat

For wheel only;

hwheel=Iwheelωwheel

In body frame:

hsat=ωxIxωyIy

ωwheel=ωwheelsat+ωwheelcarth

hwheel=J1 0 00 J2 0

ωwheel=0Ω

hwheel=hwheelbody+ω⋅hwheel

hwheel=ωxJ1(ωy+Ω)J2

Combining both and taking the time derivative of it gives us the Moments on the satellite.

M2=ωx(Ix+J1)+ωyωz(J1−J2)−ωzΩJ2ωyIx+(ωy+Ω)J2

Equation above gives the full dynamics of the satellite it should be noted that the pitch dynamics is independent from the roll and yaw dynamics whereas roll and yaw dynamics are coupled.

## 3.5 Angular Velocities and Euler Angles

The control of the satellite is managed by the control of the Euler angles thus a relation between the angular velocities and Euler angle is necessary to obtain. The angular velocities are due to motion of the frame [1] with respect to frame [0] and motion of frame [2] with respect to frame [1].

[0]

[1]

0t

U2(0)

Fig2-3 Transformation from frame [0] to frame [1]

In order to obtain a relation transformation from frame [0] to frame [1] defined. Then the transformation matrix from frame [0] to frame [1].

R(0,1)=cos(ω0t)     0         sin(ω0t)0              1            0

Similarly the transforms in frame [1] to frame [2] can be visualized.







[1]

[a]

[b]

[2]

U1(1)

U2(a)

U3(b)

Fig 2-4 Transformation from frame [1] to frame [2]

The frames [a] and [b] are just the intermediate steps. The figure means that first rotate the frame about the first unit vector with angle of and form frame [a] then rotate it about second unit vector with angle of  and form frame [b], finally rotate it about the third unit vector with an angle  and form frame [2]. Since it is assumed that the rotations that takes place sequentially the transformation matrices can be written as:

R(1,a)=1           0                00      cos(φ)      -sin(φ)

Then the angular velocity of frame [2] with respect to frame [0] can be written as:

ω(2)=φU1(1)+θU2(a)+ψU3(b)+ω0U2(0)

And representation of it in body frame is

ω(2)=φU1(12)+θU2(a2)+ψU3(b2)+ω0U2(02)

Ui(jk) means image of ith unit vector of frame [j ] in frame [k]Since the second unit vectors of frame [0] and frame [1] are coincide then

U2(02)=U2(12)Since the transformation matrix for frame [0] to frame [1] is time dependent, it would be hard to deal with it.

Similarly last rotation takes place about third unit vector so it coincides in frames [b] and [2].

U3(b2)=U3(22)Finally the angular velocity

ω(2)=φU1(12)+θU2(a2)+ψU3(22)+ω0U2(12)

The following transformations:

U1(12)=R(2,1)U1(11)       {R(2,1)=R(1,2)T      {U1(11)=100TU2(a2)=R(2,a)U2(aa)      {R(2,a)=R(a,2)T      {U2(aa)=010TU3(22)=  001TU2(12)=R(2,1)U1(11)      {R(2,1)=R(1,2)T      {U2(11)=010T

U1(12)=R(2,1)U1(11)       {R(2,1)=R(1,2)T      {U1(11)=100TU2(a2)=R(2,a)U2(aa)      {R(2,a)=R(a,2)T      {U2(aa)=010TU3(22)=  001TU2(12)=R(2,1)U1(11)      {R(2,1)=R(1,2)T      {U2(11)=010T

After putting the values

Then

Similarly

U2(12)=sin(φ)sin(θ)cos(ψ)+cos(φ)sin(ψ)−sin(φ)sin(θ)sin(ψ)+cos(φ)cos(ψ)

U1(12)=cos(θ)cos(ψ)−cos(θ)sin(ψ)

U2(a2)=sin(ψ)cos(ψ)

Finally

ωxωyAfter substituting the above equation and its time derivative in the torque equation the Equation of motion will be obtained.

## 3.6 Simplified Equations

Since the above equations highly non-linear and coupled it is too hard to generate an algorithm to control the system. Therefore some simplifications are inevitable. The Euler angles can be written in the following form

φ=φ0+δφθ=θ0+δθψ=ψ0+δψIn these equations equilibrium row, pitch, yaw angles respectively and other terms are the deviations or the disturbances from this point. If the body frame were set at the equilibrium position then all values would be zero and following equations left.

φ=δφθ=δθψ=δψSince these angles assumed to be small their sines would be zero and cosines would be 1.

And also the product of these small numbers would be much smaller and would be ignored.

Then

R(1,a)=1         0         00         1     -δφ

Then other related equations become

U2(12)=δψ1

After further manipulations the equation becomes.

ωx=ω0δψ+δφωy=ω0+δθωz=−ω0δφ+δψThe simplified equations of motion can easily be obtained by substituting these and their derivatives in torque equations.